Embedded Coding of Still Images Using

Block Transforms

Radoslav Vargic

Department of Telecommunications,

FEI STU Bratislava, Slovakia

Transform coding of images:

1. Classical transform coders with non-embedded code (JPEG,…)

2. Progressive transmission  coders - embedded code (EZW, SPIHT,…)

Properties of an Embedded coder

- output is bit stream, where bits are generated in order of their importance

- all codings of the same image at lower bit rates are embedded in the beginning of the output bit stream for the target bit rate

Wavelet approach and SPIHT

- EZW(Embedded Zerotree Wavelet coder )  by J.Shapiro

- Improved EZW by effective bitplane coding - SPIHT (set partitioning in hierarchical trees)  by A.Said, D.Pearlman

Construcion: DWT   à  EZW bit plane coding à  arith. coding

Modifications to SPIHT

- Use of block transforms with various block sizes (8,16,32,64) with spectral reordering(2 ways investigated)

DWT                 BT                       DWT                 BT

a) standard reordering       b) zig-zag reordering

- Recursion of block transform on lowest LP band

- Various block transforms used (Slant transform, Fourier transform with reordering, Walsh transform with reordering, Hartley transform, …)

- Structure with deblocking algorithm proposed (whole decoding/coding process is asymetrical)

Replacement of DWT by Block Transform in SPIHT

Block Transform

M x M

Coeff. reor-dering

De-

blockig

Estimation

Block Transform

M x M

Coeff.

reor-

dering

a) Forward Transform

b) Backward Transform

M=8, 16, 32

Comparison of rate-distortion performance between block transform (DCTII) with different block sizes (8x8, 16x16, 32x32) and other coders

|  | Lena 512x512, 8 bpp |  |  |  |  |  |  |  |  | Barbara 512x512, 8 bpp |  |  |  |  |  |  |  |  | Goldhill 512x512, 8 bpp |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rate (bpp) | SPIHT | *Xiong | 8x8 | 16x16 | 32x32 | SPIHT | *Xiong | 8x8 | 16x16 | 32 x 32 | SPIHT | 8x8 | 16x16 | 32x32 |
| 1 | 40.4 | 39.62 | 39.63 | 40.1 | 40.04 | 36.41 | 36.1 | 36 | 37.26 | 37.48 | 36.55 | 35.97 | 36.48 | 36.52 |
| 0.75 | 39.04 | 38.06 | 38.03 | 38.66 | 38.64 | 34.25 | 33.7 | 33.56 | 35.11 | 35.38 | 34.95 | 34.38 | 34.87 | 34.92 |
| 0.5 | 37.21 | 36 | 35.85 | 36.73 | 36.78 | 31.39 | 30.82 | 30.61 | 32.33 | 32.73 | 33.13 | 32.35 | 33.02 | 33.11 |
| 0.25 | 34.11 | 32.25 | 31.74 | 33.34 | 33.56 | 27.58 | 26.83 | 26.57 | 28.44 | 29.07 | 30.56 | 29.41 | 30.33 | 30.54 |

| Comparison of rate-distortion performance between block transform (DCT II) with different block sizes (8x8, 16x16, 32x32) and other coders |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| a)Lena |  | b)Barbara |  |  |  |
| Improvement of coding process via performing transform on DC coefficients |  | Best results of Slant, Walsh, and Fourier transforms when coupled with SPIHT |  |  |  |
| a) | b) |  |  | c) | d) |
| e) | f) |  |  | g) | h) |

A detail of lena image coded at 0.25 bpp, with corresponding PSNR:

a) baseline JPEG, 31.6dB b) DCT 8x8+SPIHT, 31.74dB c) DCT 16x16+SPIHT, 33.34dB d) DCT 32x32+SPIHT, 33.56dB  e) Original Image f) SS 16x16+SPIHT, 26.88dB  g) FP 32x32+SPIHT, 27.13dB h) WS 16x16+SPIHT, 26.0dB

CONCLUSION

- Reordering Type A is more efficient than B Type by 0.15dB and more (more importance of directional information over energy compaction)

- By increasing block size of BT to reasonable values (16,32) we can outperform SPIHT up to by 1.5 dB with still good visual quality  for wide range bit rates

- Recursive block transform of  lowest LP band can improve PSNR at given bit rate  up to 1dB (using usual 8x8 blocks).

- Incorporating deblocking algorithm we can decrease blocking effect. More computational effort is then only in the decoder part (asymetry)

- Using some other block transforms(with increased block size) with presented algorithm can still outperform JPEG, DCTII for smaller blocks  and for some images even original SPIHT